	 
	Сенсоры применяемые с модулями 
	 

	Температурный сенсор 
	Температурный сенсор

Температурный сенсор SMT160-30 - трух выводной температурный сенсор с duty-cycle выходом. SMT160-30 (TO18 model) имеет точность 0.7 °Cв диапазоне от-30 °C до +100 °C и точность 1.2 °C в диапазоне от -45 до +130 °C. 

Температурный пробник: SMT1603018 Probe: в металлическом тюбике 7 mm. x 40 mm. Кабель: 5 m. PVC shielded, 3.5 mm. Абсолютная точность .7 °C Линейность выхода .2 °C. 




Compatibel with 
BR16GM-SMT, BR160GM-SMT, BR160SM-SMT, BR1280GM 
	[image: image2.png]



[image: image3.png]




	 
	 
	 

	Температурный сенсор 
	Pt1000/Pt100 Temperature Sensor



Compatibel with 
BR128GM-Pt1000, BR128xGM-Pt1000
	[image: image5.jpg]




	 
	 
	 

	Current Sensor 

Current Switch
	AC Current sensors and AC Current switch 

AC Current sensors (split or solid core)

Датчики серии CTA, CTV и CS предназначены для бесконтактного измерения постоянного и переменного токов. Датчики имеют аналоговый выход, напряжение на котором прямо пропорционально величине тока, протекающего через контролируемый проводник. Выход 4...20 mA или 0...10Vdc. Все сенсоры имеют джампер для выбора диапазона 0-10, 0-20, 0-50A - CTV-50, SCTV-50 - 0-100, 0-200, 0-250A -CTV-250, SCTV-250). Эти сенсоры можно использовать для мониторинга тока в различных приложениях. CS-SD10L, CS-SP10L - для диапазонов 0...10/20/50A, CS-SD10 or CS-SP10 для диапазонов 0...100/150/200A.

AC Current switch (split or solid core)
AC токовые выключатели для аппликаций “Go/No Go”. Токовые переключатели можно использовать для мониторинга работы вентиляторов, насосов, моторов, генераторов и другого оборудования. Возможно определять неисправность и выключение оборудования. Все сенсоры доступны в неразъемном и рахъемном исполнении
CSW серия в активном состоянии с нормально открытом контактом (NO). Контакт замыкается через a solid-state switch когда ток превышает установленный уровень. CSW серия можно использовать для мониторинга устройств с с максимальным непрерывным током до 200A 
CS-F серия доступна в неразъемном и рахъемном исполнении с нормально открытым контактом (N/O). Все сенсоры имеют solid-state выход с фиксированным setpoint. Сенсоры предназначены для использования для мониторинга статуса любого типа оборудования переменного тока. 




Current sencor compatibel with
BR16GM-2A/-4A, BR160GM-2A/-4A, BR160SM-2A/-4A, 
BR128GM-4A/-Pt1000, BR1280GM 
Current switch compatibel with all our modules
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM 
	[image: image7.jpg]



[image: image8.jpg]



[image: image9.jpg]




	 
	 
	 

	Ground Fault Sensor

DC Current Switch

DC Current Transducer


DC Hall Effect Current Transducers

True RMS AC Current Transduce
	Ground Fault Sensor
DC Current Switch
DC Current Transducer

AKS-C NOAC Ground Fault Sensor, Factory adjusted set point. 
NO output. Primary current 5 - 950 mA AC. 
Для мониторинга и защиты от неисправностей и ошибок потери питания в цепях переменного тока. 
Normally Energized: защита от ошибок, неисправностей или пропадания питания. Normally Deenergyzed: защита от ошибок, неисправностей только когда питание подключено.
DKS 100 C Current Operated Switches. Input - 4-20A, 10-50A, 15-100A (with manual jumper). Hysteresis is 5% of set point. DFS 100C SDT - Relay output (SPDT) or DKS 100C NOW - Solid State Output (Normally Oupen). 
DC Current Transducer. Split or solid core. 0-5VDC output DK-B5 0-10VDC output DK-B10 proportional to measured current. For DC measurement. Jumper selectable range. 



DC Hall Effect Current Transducers
True RMS AC Current Transducer

The CR5200 Series, DC Hall Effect Current Transducers are designed to output a DC signal which is proportional to a DC current input signal. These devices are designed for direct current only, targeting them towards general and daily applications. 0-5Vdc or 4-20mA DC outputs. Supply Voltage: 24 VDC. The CR4100 Series True RMS Current Transducers and Transmitters are designed for applications where AC current waveforms are not purely sinusoidal. More precise and accurate than other transducers, these devices are ideal in chopped wave and phase fired control systems. 0-5Vdc or 4-20mA DC outputs.



Current transducer compatibel with
BR16GM-2A/-4A, BR160GM-2A/-4A, BR160SM-2A/-4A, 
BR128GM-4A/-Pt1000, BR1280GM 
Ground Fault Sensor and Current switch compatibel with 
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM 
	[image: image12.jpg]



[image: image13.jpg]



[image: image14.png]



[image: image15.png]




	 
	 
	 

	Сенсор влажности и температуры
	Сенсор влажности и температуры SHT
Digital relative humidity & temperature sensors. Fully calibrated digital output. Excellent long term stability. Fast response time. Tiny size and low energy consumption 




Compatibel with BR128GM-SHT, BR1280GM (optional)
	[image: image17.jpg]




	 
	 
	 

	Soil Moisture Probe
	Vegetronix Soil Moisture Sensor Probes
The VG400 which has an output range of 0 to 3V (Vegetronix Soil Moisture Sensor Probes) for low cost monitoring of soil water content. Supply Voltage 3.3V to 20 VDC. Output 0 to 3V nearly linear with moisture content.
(ask us for compatibilities with our modules types)
	[image: image18.jpg]




	 
	 
	 

	Soil Moisture Sensor
	Soil Moisture Sensors
The Model 250-124 Soil Moisture Sensor monitors soil moisture reliably in all types of soils at all moisture levels. Soil moisture in fields, turf, landscapes, and greenhouses may be monitored equally well. The sensor output signal may be used to directly control irrigation. The sensor reads the percent of available moisture by volume, independent of soil texture. 
Power requirements: 12 Vdc ± 20% @ 40mA Output: 4-20mA or 0-5Vdc = 0-100% soil moisture (linear) Overall size: 3/4" diameter x 27" long Shipping weight: 3 lbs Turn-on time: 1 second from power up Operating temperature range: 0-70°C 

NovaLynx Corporation
Compatibel with 
BR16GM-2A/-4A, BR160GM-2A/-4A, BR160SM-2A/-4A, 
BR128GM-4A/-Pt1000, BR1280GM
	[image: image19.jpg]




	 
	 
	 

	Tilt sensor 
	Tilt Sensor 

Sencera 603 Tilt Sensor. Tilt degree Notmally open 33° (off - on); other tilt angle are available. Tilt angle 20-45°

Compatibel with all our modules
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM
	[image: image20.jpg]




	 
	 
	 

	Сенсор давления 
	Сенсор давления
SDP1000, SDP2000 
The Sensirion differential pressure sensor with dynamic measurement principle. Measurement ranges: -5 -125 Pa / -0.02 -0.5 inch water column (SPD1000) or -100 - 3500 Pa/-0.4 - 14.0 inch water column (SPD2000). Voltage output: 0.25 to 4.0 Vdc. High resolution (0.05 Pa). High accuracy (0.2% fs). Fully calibrated and temperature compensated. 




Compatibel with 
BR16GM-2A/-4A, BR128GM, BR1280GM 
BR160GM-2A/-4A, BR160SM-2A/-4A 
	[image: image22.jpg]




	 
	 
	 

	Сенсор потока жидкости 
	OEM цифровой сенсор потока жидкости
LG16 OEM Sensor Elements, Liquid flow rates up to 5000 Jl/min. Totally non invasive, pressures up to 200 bar. 
Power supply: 2V ... 24V. Analog output 0-5V.




Compatibel with 
BR16GM-2A/-4A, BR128GM, BR1280GM 
BR160GM-2A/-4A, BR160SM-2A/-4A 
	[image: image24.jpg]




	 
	 
	 

	Сенсор влажности и температуры
	Humidity and temperature sensor

Serie EE08 humidity and temperature sensor 
with analog output (0-1V/2,5V/5V/10V).

Compatibel with 
BR16GM-2A/-4A, BR128GM, BR1280GM 
BR160GM-2A/-4A, BR160SM-2A/-4A 
	[image: image25.png]




	 
	 
	 

	Level Switch
	Level Switch 
Liquid level switches with float. 
Single Point Level Switches LS-6/LS-7 
Small Size - Engineered Plastics, Side Mount LS-7 Series - Compact side mounts are the solution to many small tanks. These low-cost units are ideal for high volume use in small tanks and vessels. Engineered plastics construction offers broad compatibility in water, oils and chemicals. The high temperature capability of Versaplast offers an alternative to high cost stainless steel switches.





Compatibel with all our modules
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM
	


 





	 
	 
	 

	Датчики уровля 
	Датчик уровня 

Liquid level switches with float. 
RSF54 manages a higher temperature and is manufactured from glass fibre stabilised polypropylene. RSF53 is manufactured from glass fibre filled nylon. Both materials have strong resistance to most liquids. Provided with 1 m cable.
Breaking capacity: 15 VA, 1 A, 50 V Temp. range RSF53: -20 to +75 °C; RSF54: -20 to +100 °C Assembly holes: ø12.2 mm; Dimensions: ø30x70 mm 

Compatibel with all our modules
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM
	[image: image29.png]




	 
	 
	 

	Сенсоры уровня и потока жидкости
	Liquid Level Sensors, Flow Swithes,
Position Sensors
Gentech Procucts range
Vertically and horizontally mounted liquid level sensors. The analoque liquid level sensor. Optical level sensor.
Horizontal and vertical mounted flow switches.




	[image: image31.jpg]



[image: image32.jpg]




	 
	 
	 

	Сенсоры утечек, потока жидкости и газа
	Детектор утечек,
Детектор уровля жидкости 
Детекторы потока жидкости и газа 
LO10 - The general purpose optic switch is a great choice for leak detection in and around secondary containment sumps, tanks and pipes. The sensor can be applied in a broad range of chemical solutions. NO / NC Output.
FT10 - Termal Dispersion Flow Switch. Adjustable set point from .04-3 fps. The general purpose liquid flow switch is an ideal choice for a wide range of relatively clean water and chemical solutions with non-coating and/or crystallizing characteristics. The reliable solution for pump and process protection. NO / NC Output.
GT10 - Termal Dispersion Flow Switch. Adjustable set point from 1-90 fps. The general purpose gas flow switch is a great choice for a broad range of relatively clean gases with non-coating characteristics. The reliable solution for filter and process protection. NO / NC Output.



	[image: image34.png]




	 
	 
	 

	Сенсор уровня и расстояния
	Сенсор уровня, сенсор расстояния

Photoelectric sensors, Inductive sensors,
Capacitive sensors, Ultrasonic sensors,
Force, strain and pressure sensors,
Magnetic sensors

Products 




	[image: image36.jpg]



[image: image37.jpg]




	 
	 
	 

	Proximity Sensor 
	Inductive Proximity sensors 

Osiprox XS sensor with NO/NC programmable output with NO/NC programmable output are inductive proximity detectors of the four-wire DC type with static output PNP + NPN.




Compatibel with all our modules
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM
	



	 
	 
	 

	Термостат
	Термостат
Temp-Stat™ - is a low cost, non-adjustable, temporary construction thermostat preset to a given temperature. Warehouse and compressor lock-out versions are also available. Available in many temperatures for both heating and cooling. 



Compatibel with all our modules
BR16GM, BR160GM, BR160SM, BR128GM, BR1280GM 
	[image: image41.png]




	 
	 
	 

	Детектор движения 
	KUBE PYR components,
Pyroelectric Sensors for Motion Detectors,
Compact Motion Detector Module
TR257 compact motion detector module; 
C172 standard TO-5 Dual Pyroelectric Sensor: 
6192 Flat Pack Dual Pyroelectric Sensor: 
7192 TwinSide Flat Pack Dual Pyroelectric Sensor for 180° wide angle motion detectors 



	[image: image43.jpg]




 INCLUDEPICTURE "http://www.bieneelectronics.com/products/images/sensors/sample-pyro.jpg" \* MERGEFORMATINET [image: image44.jpg]




	 
	 
	 


